Sukkur IBA University Kandhkot Campus

Applied Physics, BSCS-II ( Spring 2025)

Lab Handout # 02: Verification of Ohm’s Law

Instructor: Sahib Khan

Note: Submit this lab hand-out in the next lab with attached solved activities and exercises

Submission Profile

Name:

Submission date:

Enrollment ID:

Comments:

_____________________________________________________________________________

___

Instructor

Signature

Lab learning objectives:

The objective of this experiment is to use Ohm's Law to determine the resistance of several Individual resistors. You will apply various voltages across the resistor (or circuit) using a power supply and measure the current I through the resistor and the voltage V across the resistor for each setting of the power supply. From a plot of V vs. I, you will determine the resistance. You will compare the measured value of the resistance with the manufacturer's value.

Lab hardware and software required:

Variable DC power supply, Breadboard, Resistors of different values, Multimeters & connecting wires.

Background theory:

Ohm's Law can be written algebraically as ΔV = RI, where ΔV, measured in volts, represents the potential drop or potential difference across the conductor, I is the current in the conductor measured in amperes, and R is the resistance of the conductor measured in units called \ohms" symbolized by , upper-case Greek omega.

Note: Some textbooks use V rather than ΔV for potential difference.

Resistance and Resistors:

Resistance is a property of materials. Resistors are conducting devices made from materials, which satisfy Ohm's Law. If the potential difference across a resistor is set at 1 volt, and if a current of 1 amp is measured in the conductor, then its resistance is determined to be 1 ohm, or 1. Instead of using thin wires as Ohm did in his original experiment, you will replicate his results using small cylindrical ceramic resistors. You will notice colored bands on the resistors. These bands form a code that indicates the resistance of the resistor. Later we will discuss how to read this color code.

Measuring Current and Voltage

Measuring Current and Voltage Ammeters are used to measure the current owing in a circuit. To do so, the ammeter should be connected in series with the circuit element through which you want to measure the current. Introducing an ammeter into a circuit should not affect the flow of current in the circuit and, therefore, ammeters have very low resistance.

Voltmeters are used to measure the potential difference or voltage drop across a circuit element. To do this, the voltmeter should be connected to the two points across which you want to measure the potential difference. In other words, the voltmeter should be connected parallel to the circuit element. Voltmeters should not affect the current flowing through the circuit element and, therefore, voltmeters have high resistance. This prevents current from flowing through them.

Reading the Resistor Code

The resistance of most ceramic resistors can be determined from the colored bands printed On the resistor. Each color represents a digit from 0 to 9.

The first two bands indicate the mantissa of a number in scientific notation; the third indicates the power of ten. The fourth band indicates the tolerance or the uncertainty expressed as a percentage in the value of the resistance (gold: 5%, silver: 10%, no 4th band: 20%). Therefore, in order to know which end of a resistor to start from when reading the color code, it is useful to remember that the 4th band, if present, is metallic in color (gold or silver). If regular colors are present instead of these metallic bands, sometimes the color bands will be spaced differently or are closer to one end of the resistor to help indicate which end to start reading from. We will not be concerned with five-band resistors in this lab. If a fifth band were present, the first three bands indicate the mantissa, the fourth indicates the power of ten and the fifth indicates the tolerance (as brown, red, orange, yellow, or gold). See the example in Fig. 3 below.

Procedure A: Determining Resistor Value Using Color Code

- Enter the color of the four bands for the two resistors in Data Table 1.

- Determine the resistor value for the resistors and enter these values in Data Table 1. You will use these values as the expected or manufacturer's values when comparing with the experimental values.

Procedure B: Determining R1

To determine the resistance, you will set up the following circuit.

- Make sure your power supply is turned off

- Connect the circuit as shown in the figure above Precaution: If the multi meters are not set to the proper scale, it can damage the meters.

- Set the power supply to deliver 3 V. Record the ammeter and voltmeter readings on your Worksheet. Record the actual voltmeter reading and not the value on the power supply as these two readings might be slightly different.

- Increase the power supply output in steps of 1 volt and record the ammeter and voltmeter readings on your worksheet, for a total of five different voltmeter readings.

- Construct a graph of ΔV versus I using Excel.

- Compute the percent error between the manufacturer's value of the resistance (from Data Table 1) and the experimental value.

Procedure C: Determining R2

- Disconnect the first resistor and connect the second resistor

- Repeat the steps 3-8.

- Compute the percent error between the manufacturer's value of the resistance (from Data Table1) and the experimental value.

Worksheet

As you work through the steps in the lab procedure, record your experimental values and the Results on this worksheet. Use the exact values you record for your data to make later calculations.

Procedure A: Reading the color code of unknown resistors.

Table 1

| Resistor | Band 1 |  | Band 2 |  | Band 3 |  | Resistance |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Color | Value | Color | Value | Color | Value | Ω |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |

Table 2

| Trial | V (V) | Current (A) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |

Draw plot of voltage versus current.

Answer the following questions.

Q.1: What is Potential difference?

Q.2: What is Electric Power?

Q.3: Why are resistors color coded?

Q.4: Define the following terms

  - Resistance

  - Resistivity

  - Current

  - Electric potential

Q.5: Enlist and explain the types of resistors.