Sukkur IBA University Kandhkot Campus

Department of Computer Science

Applied Physics, Spring 2025

Lab Handout # 04: PARALLEL CIRCUITS

Instructor: Engr. Sahib Khan

Name: __________________________________ Date: ________________________

Score: __________________________________Signature of Instructor___________

PERFORMANCE OBJECTIVE:

- To learn the basic concepts and relationships of current and voltage measurements in DC circuits containing resistors wired in Parallel configuration.

- To understand and use Ohm’s Law applied to Parallel Circuits.

- To learn the relationships of the total resistance of resistors connected in Parallel Circuits.

- To learn to use ammeters, voltmeters, ohmmeters, and multimeters to properly measure voltages, currents, and resistances.

EQUIPMENT AND COMPONENTS

- Multimeter

- Power supply

- Connecting Leads

- Breadboard

- Few Resistors

- Ohmmeter

- Voltmeter

- Ammeter

Introduction

Most common household electrical circuits are made of many devices connected in parallel. Each device is hooked to the power source in parallel with all the other devices, each connected to the same voltage source and availing itself of the same voltage. Each device has its own characteristic resistance, and therefore each draws from the source a different amount of current, depending on its resistive value. While the voltage being accessed is nearly the same for all devices, the amount of current drawn from the source increases as each device draws its respective current based on its resistance. As a result as more and more devices are connected in parallel, the total amount of current drawn from the source increases. It thus has the effect of causing the resistance to decrease with each additional resistance added. Additional devices added to a circuit, require additional current from the source until something is overloaded. More current is required beyond that which can be supplied by the source or carried by the conductors without burning up.

Theory

Ohm’s Law

Ohm’s Law is the relationship between the current I flowing through a resistance R and the potential drop across it V. The current is directly proportional to the potential difference across the resistance and is inversely proportional to the resistance,

I=V/R 				(1)

As an alternative, Ohm’s Law may be stated as: The potential difference V across a resistance is directly proportional to the current I flowing through the resistance and the resistance R, or

V=I ×R			(2)

Ohm’s Law can be rearranged to define the resistance R so that

R=V/I				(3)

If the potential difference across the resistance is measured in volts (V) and the current flowing through the resistance is measured in amperes (A), then the resistance values will be in units of ohms.

Resistors in Parallel

Resistors are connected in parallel when one end of each resistor is connected to a common point and each of their other ends is connected to another common point as shown in Figure 1. The current IT that is supplied by the battery is divided into 3 separate currents, I1, I2, and I3, each flowing through resistors R1, R2, and R3 respectively. After flowing through the resistors, the 3 currents rejoin into a common current, IT, or that current flowing back to the battery. The total current flowing in the circuit is the sum of the separate currents flowing through the resistors,

IT = I1 + I2 + I3 				(4)

Figure 1: Three resistors R1, R2, and R3 connected in parallel showing 						the flow of current.

From Equation 1 the total current is related to the total voltage and total resistance by:

IT = VT / RT 			(5)

and the current, potential difference, and resistance of each separate resistor is given by:

I1 = V1 / R1 			(6)

I2 = V2 / R2 			(7)

I3 = V3 / R3 			(8)

Substituting these equations into Equation 4

VT/RT =V1/R1+V2/R2+V3/R3 	(9)

The voltage drops across the battery and resistors are all equal, and as illustrated in Figure 2

Figure 2: Three resistors R1, R2, and R3 connected in parallel showing the potential differences all being equal.

VT=V1+V2+V3 			(10)

Since all the potentials are equal Equation 9 reduces to:

(11)

Which is the equation for the total resistance of 3 resistors connected in parallel, As was the case for series resistors, the total resistance of any number N (N≥2) of resistors connected in parallel can be found using the same general procedure and will result in the relationship,

(12)

In circuits with combinations of resistors in series and parallel, the total resistance can be found by breaking the circuit down into its simplest unit consisting of either resistor in series or parallel and then adding its total resistance back into the next simplest unit and finding its total resistance, and so on until the entire circuit has been reconstructed.

PROCEDURE:

1.With an ohmmeter measure the each resistor value for the resistors required for this experiment, take resistances of your own choices available. Connect the resistors in parallel and measure the total resistance RT. Record the results in Table 1.

2.Using the information given in Theory and Figure 1 & 2, calculate the total resistance RT, the Parallel VT, and the current through R1, R2, R3. Record the result in Table 2. Take voltage and resistances of your choice available in the Lab.

3.Connect the circuit shown in Figure 1 and turn on the DC Power Supply, adjust the power supply level to 10V DC.

4.Measure the current through each resistance as indicated in the Figure 1 and record the result in Table 2.

5.Repeat the procedure for 15V DC and fill the Table 3.

6.After recording the measured current through resistors, compare the measured and calculated values. Use the formula for determining the percentage.

%=Difference between Measured and Calculated × 100

Calculated

TABLE 1

| S.NO | Calculated Resistance(Nominal) | Tolerance Range | Measured Resistance |
| --- | --- | --- | --- |
| 1 | R1 |  |  |
| 2 | R2 |  |  |
| 3 | R3 |  |  |
| 4 | RT |  |  |

TABLE 2

| Value | Calculated | Measured | % Error |
| --- | --- | --- | --- |
| RT |  |  |  |
| VT |  |  |  |
| IR1 |  |  |  |
| IR2 |  |  |  |
| IR3 |  |  |  |

TABLE 3

| Value | Calculated | Measured | %Error |
| --- | --- | --- | --- |
| RT |  |  |  |
| VT |  |  |  |
| IR1 |  |  |  |
| IR2 |  |  |  |
| IR3 |  |  |  |

CONCLUSION