LAB EXPERIMENT NO # 04

Name: Hussain Ahmed                                       Date: 4-may-2026

Score: __________________________________Signature of Instructor___________

SERIES CIRCUITS

PERFORMANCE OBJECTIVE:

- To learn the basic concepts and relationships of current and voltage measurements in DC circuits containing resistors wired in Series Circuits.

- To understand and use Ohm’s Law applied to Series Circuits.

- To learn the relationships of the total resistance of resistors connected in Series Circuits.

- To learn to use ammeters, voltmeters, ohmmeters, and multimeters to properly measure voltages, currents, and resistances.

EQUIPMENT AND COMPONENTS

- Multimeter

- Power supply

- Connecting Leads

- Breadboard

- Few Resistors

- Ohmmeter

- Voltmeter

- Ammeter

Introduction

Most common household electrical circuits are made of many devices connected in parallel. Each device is hooked to the power source in parallel with all the other devices, each connected to the same voltage source and availing itself of the same voltage. Each device has its own characteristic resistance, and therefore each draws from the source a different amount of current, depending on its resistive value. While the voltage being accessed is nearly the same for all devices, the amount of current drawn from the source increases as each device draws its respective current based on its resistance. As a result as more and more devices are connected in parallel, the total amount of current drawn from the source increases. It thus has the effect of causing the resistance to decrease with each additional resistance added. Additional devices added to a circuit, require additional current from the source until something is overloaded. More current is required beyond that which can be supplied by the source or carried by the conductors without burning up.

Theory

Ohm’s Law

Ohm’s Law is the relationship between the current I flowing through a resistance R and the potential drop across it V. The current is directly proportional to the potential difference across the resistance and is inversely proportional to the resistance,

I=V/R 				(1)

As an alternative, Ohm’s Law may be stated as: The potential difference V across a resistance is directly proportional to the current I flowing through the resistance and the resistance R, or

V=I ×R			(2)

Ohm’s Law can be rearranged to define the resistance R so that

R=V/I				(3)

If the potential difference across the resistance is measured in volts (V) and the current flowing through the resistance is measured in amperes (A), then the resistance values will be in units of ohms.

Resistors in Series

Figure 1 shows 3 resistors, R1, R2, and R3, connected in series in a closed circuit powered by a single battery or Emf source. In this circuit the current supplied by the battery flows through each resistor, with the current in each resistor being the same. If the current supplied by the battery is IT, the current in each resistor is I1, I2, and I3, and they are all one and the same, then

IT = I1 = I2 = I3 				(4)

The voltage drop across the battery VT will be the total sum of the individual drops across each of the 3 resistors, and

VT=V1+V2+V3 			(5)

Where V1 is the potential difference across R1, V2 is the potential difference across R2, and V3 is the potential difference across R3. From Equation 2,

VT=IT × RT 				(6)

V1=I1 × R1 				(7)

V2=I2 × R2 				(8)

V3=I3 × R3 				(9)

Substituting these equations into Equation 5 gives

IT RT = I1 R1 + I2 R2 + I3 R3 		(10)

Since 	IT = I1 = I2 = I3

RT = R1 + R2 + R3 			(11)

Therefore, when resistors are connected in series, the total resistance is just the sum of the individual resistances. While this has been shown for 3 resistors, the total resistance of any number of N (N≥2) of resistors connected in series, end to end, can be found using the same general procedure. Therefore for resistors connected in series

(12)

PROCEDURE:

1.With an ohmmeter measure the each resistor value for the resistors required for this experiment, take resistances of your own choices available. Connect the resistors in series and 2.measure the total resistance RT. Record the results in Table 1.

3.Using the information given in Theory and Figure 1, calculate the total resistance RT, the series current IT, and the IR voltage drop across R1, R2, R3. Record the result in Table 2. Take voltage and resistances of your choice available in the Lab.

4.Connect the circuit shown in Figure 1 and turn on the DC Power Supply, adjust the power supply level to 10V DC.

5.Measure the IR voltage drop across each resistance as indicated in the Figure 1 and record the result in Table 2.

6.Repeat the procedure for 15V DC and fill the Table 3.

7.After recording the measured voltage drops, compare the measured and calculated values. Use the formula for determining the percentage.

%=Difference between Measured and Calculated × 100

Calculated

TABLE 1

| S.NO | Calculated Resistance(Nominal) | Tolerance Range | Measured Resistance |
| --- | --- | --- | --- |
| 1 | R1 | 5% | 100 ohms |
| 2 | R2 | 5% | 200 ohms |
| 3 | R3 | 5% | 300 ohms |
| 4 | RT | 5% | 600 ohms |

TABLE 2

| Value | Calculated | Measured | % Error |
| --- | --- | --- | --- |
| RT | 600 ohms | 600 ohms | 0% |
| IT | 0.01667A | 16.67mA | 0% |
| VR1 | 1.67V | 1.667V | 0% |
| VR2 | 3.334V | 3.33V | 0.1% |
| VR3 | 5V | 5V | 0% |

TABLE 3

| Value | Calculated | Measured | %Error |
| --- | --- | --- | --- |
| RT | 600 ohms | 600 ohms | 0% |
| IT | 25mA | 25mA | 0% |
| VR1 | 2.5V | 2.5V | 0% |
| VR2 | 5V | 5V | 0% |
| VR3 | 7.5V | 7.5V | 0% |

CONCLUSION

The experiment was done successfully to study a series circuit. The total resistance was equal to all resistances added together, and it was bigger than the biggest resistor in the circuit. The current was the same at every point in the circuit. The voltage was shared between the resistors — each resistor got voltage according to its size. The values we measured were almost the same as the values we calculated. So, the experiment proves how a series circuit works and also proves Ohm's Law. Small differences in values happened because of resistor tolerance and small mistakes in measurement.