Sukkur IBA University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 05: Comparator

Instructor: Muhammad Ismail Qureshi

Submission Profile

Name:							Submission date (dd/mm/yy):

CMS ID:						Marks obtained:

Comments:						Receiving authority name and signature:

________________________________________________________________________________

Instructor Signature

Note: Submit this lab hand-out on LMS/e-learning before the next lab with attached solved activities and exercises.

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify Comparator operations using 74LS85 IC & NI ELVIS Kit

Lab Hardware and Software Required:

- 74LS85IC (Comparator)

- NI ELVIS Kit

- Breadboard

- Connecting Wires

Background Theory:

Comparator:

Comparator is a combinational logic circuit that compares two input binary numbers and signals if the two numbers are equal, or if one is higher than the other. The basic function of comparator is to compare the magnitude of two quantities to determine the relationship of those quantities. A binary comparator usually has three outputs: A=B, A>B and A<B. A 1-bit comparator compares two 1-bits (A & B) and determines whether they are “greater than” or “less than” or “equal to” each other by a high level on the appropriate output. As the outputs exclusively depend on the temporary inputs, the circuit is composed by logic gates. The logic symbol and logic diagram for 1-bit comparator as shown in Fig 5.1 (a & b).

Fig 5.1: (a) Logic Symbol 				(b) Logic symbol for 1-bit Comparator

74LS85 Comparator IC:

The TTL 74LS85 IC is a four bit magnitude comparator which will perform comparison of two 4-bit binary numbers. The pin configuration of IC is shown in Fig 5.2. This IC consists of eight comparing inputs (A0, A1, A2, A3, B0, B1, B2, B3), three cascading inputs (IA<B, IA=B, and IA>B). This device compares two 4-bit words (A & B) and determines whether they are “greater than”,” less than” or “equal to” each other by a high level on the appropriate output. For words greater than 4-bits, units can be cascaded by connecting the outputs (OA<B,OA=B,  and OA>B) of the least significant stage to the cascaded inputs (IA<B, IA=B, and IA>B) of the next significant stage. In addition the least significant stage must have a high level (VIN (1)) applied to the A=B input and low level voltage (VIN (0)) applied to A>B and A<B inputs.

Fig 5.2: 74LS85 Magnitude Comparator IC pin configuration

Lab Examples:

Implementation of 1-bit Comparator:

- Carry out the pin configuration of 74LS85 IC

- Take 74LS85 IC and insert it in the breadboard present on the NI ELVIS-II kit

- Apply power supply to it

- Connect inputs A and B to the data switches SW0 and SW1 respectively

- Connect the three output to the LEDs

- Analyse the circuit behaviour by carrying out different combinations with the data switches and fill the observation table for comparator

Table 5.1: Observation table of 1-bit Comparator

| Input |  | Output (A>B) |  | Output (A=B) |  | Output (A<B) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | LED (on / off) | Level ( 1 / 0 ) | LED (on / off) | Level ( 1 / 0 ) | LED (on / off) | Level ( 1 / 0 ) |

Lab Activities:

Implementation of 4-bit comparator using 74LS85:

- Connect the inputs related to binary numbers A to four switches and the inputs related to input B to other four switches

- Connect inputs IA<B, IA=B, IA>B to ground

- Connect outputs OA<B, OA=B, OA>B to three LEDs.

- With the switches compose all combinations between A & B and verify the comparator operations

Table 5.2: Observation table of 4-bit Comparator

| Input |  |  |  |  |  |  |  | Output |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4-bit Word A |  |  |  | 4-bit Word B |  |  |  | (A>B) | (A=B) | (A<B) |
| A0 | A1 | A2 | A3 | B0 | B1 | B2 | B3 | LED (on / off) | LED (on / off) | LED (on / off) |

Lab Exercises:

Design and Implementation of 8-bit comparator using 74LS85:

Description:

Schematic Diagram: