Sukkur IBA University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 07: BCD-to-7-Segments Decoder

Instructor: Muhammad Ismail Qureshi

Submission Profile

Name:							Submission date (dd/mm/yy):

CMS ID:						Marks obtained:

Comments:						Receiving authority name and signature:

________________________________________________________________________________

Instructor Signature

Note: Submit this lab hand-out on LMS/e-learning before the next lab with attached solved activities and exercises.

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify BCD-to-7 Segment Decoder operations using 74LS47 IC & NI ELVIS Kit

Lab Hardware and Software Required:

- 74LS47 IC (BCD-to-7 Segment Decoder)

- NI ELVIS Kit

- Breadboard

- Connecting Wires

Background Theory:

BCD-TO-7 SEGMENT DECODER:

Most digital equipment has some means for displaying information in a form that can be understood readily by user. This information is often numerical data but can be also alphanumeric (numbers and letters). One of the simplest and most popular methods for displaying numerical digits use a 7-segment configuration as shown in fig 7.1 (a) to form the decimal character 0 through 9 and sometimes the hexadecimal characters A through F. One common arrangement uses light emitting diodes (LEDs) for each segment. By controlling the current through each LED, some segment will lit up and others will be dark so that desired character pattern will be generated. Fig 7.1 (b) shows the segment patterns that are used to display the various digits. For example to display “6”, the segment A, C, D, E, F and G are made bright while B is dark.

Fig 7.1: (a) 7-Segment Arrangement (b) Active segment for each digit

A BCD-to-7 segment decoder is used to take a four bits BCD input and provide the output that will pass current through the appropriate segments to display the decimal digit. The logic for this decoder is more complicated than the logic of decoders because each output is active for more than one combination of input, for example the E segment must be activated for any of the digits 0, 2, 6 and 8, which means whenever any of the codes 0000, 0010, 0110 or 1000 occurs.

74LS47 BCD-TO-7 SEGMENT DECODER IC:

Fig 7.2 shows the pin-out for TTL 74LS47 IC. The 74LS47 accepts four lines of BCD input data, generates their complements internally and decodes the data with seven AND/OR gates having open collector outputs to drive indicator segments directly. RBI is active –low ripple bank input. BI/RBO is active low blanking input or ripple blanking output, A to G are active low segment outputs and A0-to-A3 are BCD inputs. The active low input terminal LT (lamp test) is used to carry out a control of the display, turning all segments contemporarily ON, while the terminals RB (ripple blanking) are used to keep the display OFF, when they should display some non-significant 0.

Fig 7.2: 74LS47 BCD-to-7 Segment Decoder IC

Fig 7.3 shows a TTL 74LS47 BCD-to-7 segment decoder being used to drive 7-segment common anode LED readout. Each segment consists of one or two LEDs. The anodes of the LEDs are all tied to VCC (+5V). The cathodes of LEDs are connected through current limiting resistors of value 330Ω to the appropriate output of the decoder. The value of limiting resistor can be found by knowing that the voltage drop across an LED is 1.7V and it takes approximately 10 mA to illuminate it. We need seven 330Ω resistors (eight if the decimal point is included) for current limiting. Dual-in-line package (DIP) resistor networks are available and simplify the wiring process because all seven (or eight) are in single DIP. The decoder has active low outputs.

Fig 7.3: BCD to 7-segment decoder driving a common anode 7-segment LED display

To illustrate the operation of this circuit let us suppose that the BCD input is A3=0, A2=1, A1=0, A0=1, which is BCD for 5.With these inputs the decoder output A,F,G,C and D will be driven low (connected to ground) allowing the numeral 5. The B and E outputs will be HIGH (open), so that the LED segments B and E will not conduct.

Lab Examples:

Implementation of BCD-to-7-Segment decoder:

- Carry out the pin configuration of 74LS47 IC

- Take 74LS47 IC and insert it in the breadboard present on the NI ELVIS-II kit

- Apply power supply to it

- Connect the inputs A0, A1, A2 and A3 of decoder to switches.

- Connect the outputs of decoder “A to G” to the corresponding inputs of LED display

- Set all combinations of inputs with help of switches, check the LEDs and fill the observation table

Note: The outputs of the decoder are negated logic, so a LED ON means a LED of the 7-segment display off.

Table 7.1: Observation table of BCD-to-Segment decoder

| Input |  |  |  | Output |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A3 | A2 | A1 | A0 | A | B | C | D | E | F | G |

Lab Activities:

Implement this circuit using Multisim and attach that circuit diagram showing last digit of your CMS ID.

Lab Exercises:

Describe functions of LT, RBI, RBO and BI

Why do we use common anode seven segment display?

Draw pin configuration of seven segment common anode and common cathode display.