Sukkur IBA University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 09: Demultiplexer

Instructor: Muhammad Ismail Qureshi

Submission Profile

Name:							Submission date (dd/mm/yy):

CMS ID:						Marks obtained:

Comments:						Receiving authority name and signature:

________________________________________________________________________________

Instructor Signature

Note: Submit this lab hand-out on LMS/e-learning before the next lab with attached solved activities and exercises.

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify Demultiplexer operations using 74LS155 IC & NI ELVIS Kit

Lab Hardware and Software Required:

- 74LS155 IC (1-to-4 Demultiplexer)

- NI ELVIS Kit

- Breadboard

- Connecting Wires

Background Theory:

Demultiplexer:

A Demultiplexer (DEMUX) is a device which essentially performs the opposite operation of the multiplexer; it takes a single input and distributes it over several outputs. It is also called as a data distributor. The De-multiplexer is a logic device used for binary transmission of serial type, i-e data coming from a single line, on one of the N output lines of the circuit, and among these, in particular the one selected with the address. Fig 9.1 (a & b) shows the logic symbol and logic diagram for the 1-line-to- 2-line De-multiplexer circuit. In this case data input is transmitted only to one of outputs as determined by the select input code.

Fig 9.1: (a) Logic Symbol 		(b) Logic Diagram for a 1-to-2 De-multiplexer

74LS155 DEMULTIPLEXER IC:

The TTL 74LS155 is a-output Demultiplexer. Inside it has two identical decoding circuits, from 1 to 4 lines, with individual strobe commands (inhabitation) and common inputs of the binary addresses. The individual strobes enable the activation or inhabitation of each of the two 4-bit sections. The data across the input IC appear inverted across the output, while the data across 2C is not inverted. The pin-out of the 74LS155 IC is shown in Fig 9.2:

Fig 9.2: 74LS155 De-MUX IC

Lab Examples:

Implementation of 1-line-to-2 Demultiplexer:

- Carry out the pin configuration of 74LS155 IC

- Take 74LS155 IC and insert it in the breadboard present on the NI ELVIS-II kit

- Apply power supply to it

- Connect inputs S and D to two switches SW0 and SW1

- Connect the output Y0 & Y1 to the LEDs

- Compose the two possible combinations with the switch S and check that the state variations of the line D (data) are directed to the selected output and fill the observation table.

Table 9.1: Observation table of 1-line-to-2 De-Multiplexer

| Input |  | Output (Y) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Select Input (Switch S) | Input Selected (Data) | Y0 |  | Y1 |  |
|  |  | LED (on / off) | Level ( 1 / 0 ) | LED (on / off) | Level ( 1 / 0 ) |
| 0 | D0 |  |  |  |  |
| 1 | D1 |  |  |  |  |

Lab Activities:

Implementation of 1-line-to-4 De-Multiplexer:

- Connect the DATA and select inputs A, B to switches

- Connect enable input EN to ground

- .Connect the outputs Y0 to Y3 to LEDs

- Carry out the four combinations with the switches A & B and check each time on which output line the input data is present.

Table 9.2: Observation table of 1-line-to-4 De-Multiplexer

| Input |  |  | Output (Y) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Select Input |  | Input Selected (Data) | Y0 | Y1 |  | Y2 | Y3 |
| A | B |  | LED (on / off) | LED (on / off) |  | LED (on / off) | LED (on / off) |

Lab Exercises:

What is demultiplexer?

Using 74LS155 IC, make demultiplexer circuit in Multisim and attach that circuit in here. Also write down circuit details. Circuit output