Sukkur IBA University

Department of Computer Science

ESE-201: Digital Logic Design Lab

Lab # 09: Demultiplexer

Instructor: Muhammad Ismail Qureshi

Submission Profile

Name:		Ghulam Mustafa			Submission date 25/12/2025

CMS ID:	023-24-0513				Marks obtained:

Comments:						Receiving authority name and signature:

________________________________________________________________________________

Instructor Signature

Note: Submit this lab hand-out on LMS/e-learning before the next lab with attached solved activities and exercises.

Lab Learning Objectives:

Upon successful completion of this experiment, the student will be able:

- To implement and verify Demultiplexer operations using 74LS155 IC & NI ELVIS Kit

Lab Hardware and Software Required:

- 74LS155 IC (1-to-4 Demultiplexer)

- NI ELVIS Kit

- Breadboard

- Connecting Wires

Background Theory:

Demultiplexer:

A Demultiplexer (DEMUX) is a device which essentially performs the opposite operation of the multiplexer; it takes a single input and distributes it over several outputs. It is also called as a data distributor. The De-multiplexer is a logic device used for binary transmission of serial type, i-e data coming from a single line, on one of the N output lines of the circuit, and among these, in particular the one selected with the address. Fig 9.1 (a & b) shows the logic symbol and logic diagram for the 1-line-to- 2-line De-multiplexer circuit. In this case data input is transmitted only to one of outputs as determined by the select input code.

Fig 9.1: (a) Logic Symbol 		(b) Logic Diagram for a 1-to-2 De-multiplexer

74LS155 DEMULTIPLEXER IC:

The TTL 74LS155 is a-output Demultiplexer. Inside it has two identical decoding circuits, from 1 to 4 lines, with individual strobe commands (inhabitation) and common inputs of the binary addresses. The individual strobes enable the activation or inhabitation of each of the two 4-bit sections. The data across the input IC appear inverted across the output, while the data across 2C is not inverted. The pin-out of the 74LS155 IC is shown in Fig 9.2:

Fig 9.2: 74LS155 De-MUX IC

Lab Examples:

Implementation of 1-line-to-2 Demultiplexer:

- Carry out the pin configuration of 74LS155 IC

- Take 74LS155 IC and insert it in the breadboard present on the NI ELVIS-II kit

- Apply power supply to it

- Connect inputs S and D to two switches SW0 and SW1

- Connect the output Y0 & Y1 to the LEDs

- Compose the two possible combinations with the switch S and check that the state variations of the line D (data) are directed to the selected output and fill the observation table.

Table 9.1: Observation table of 1-line-to-2 De-Multiplexer

| Input |  | Output (Y) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Select Input (Switch S) | Input Selected (Data) | Y0 |  | Y1 |  |
|  |  | LED (on / off) | Level ( 1 / 0 ) | LED (on / off) | Level ( 1 / 0 ) |
| 0 | D0 | Off | 0 | On | 1 |
| 1 | D1 | On | 1 | Off | 0 |

Lab Activities:

Implementation of 1-line-to-4 De-Multiplexer:

- Connect the DATA and select inputs A, B to switches

- Connect enable input EN to ground

- .Connect the outputs Y0 to Y3 to LEDs

- Carry out the four combinations with the switches A & B and check each time on which output line the input data is present.

Table 9.2: Observation table of 1-line-to-4 De-Multiplexer

| Input |  |  | Output (Y) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Select Input |  | Input Selected (Data) | Y0 | Y1 |  | Y2 | Y3 |
| A | B |  | LED (on / off) | LED (on / off) |  | LED (on / off) | LED (on / off) |
| 0 | 0 | 1 | Off |  | On | On | On |
| 0 | 1 | 1 | On |  | Off | On | On |
| 1 | 0 | 1 | On |  | On | Off | On |
| 1 | 1 | 1 | On |  | On | On | Off |

Lab Exercises:

What is demultiplexer?

Ans: A demultiplexer (DEMUX) basically reverses the multiplexing function. It takes digital information from one line and distributes it to a given number of output lines. For this reason, the demultiplexer is also known as a data distributor. As you will learn, decoders can also be used as demultiplexers.

Using 74LS155 IC, make demultiplexer circuit in Multisim and attach that circuit in here. Also write down circuit details. Circuit output